Product Overview

Daytime Visibility Range: 1000m ~ 5000m

Nighttime Visibility Range: 500m ~ 1000m

Detection Range: 150m ~ 5000m

Product Features: High Speed & Agile, Artificial Intelligence, No Blind Spots, Dual-Spectrum

Application Areas: Vehicle Patrol, Safety Production, Urban Surveillance, Power Energy, Roads & Bridges, Forest Fire Prevention, Ecological Protection, Water Conservancy & Lakes, Ports & Docks, Barracks, Bases, etc.

Features

- Dual-Spectrum: Combines thermal imaging and high-definition visible light for complementary detection, enabling round-the-clock, wide-area rapid search and close-range clear target observation

- Equipped with electronic and optical defogging, unaffected by sandstorms, fog, rain, snow, etc

- Visible light camera features image enhancement, starlight technology, supports image stabilization, heat haze suppression, stable image under vibration, clear image under intense sun, no crawling texture

- Thermal imager features image detail enhancement, black-hot/white-hot polarity switching, multiple pseudo-color palettes

- Supports absolute positioning, relative positioning, and GPS coordinate positioning; compatible with radar and other detection devices, reducing system debugging time

- During radar linkage, there is no autofocus process, allowing the target image to appear clear immediately, facilitating target tracking

- Predictive PID algorithm for moving targets, seamless integration with radar tracking, solves the problem of tracking targets in radar blind spots

- Actively uploads azimuth angle, field of view, focal length value, and focus value

- Supports stepless speed control, speed resolution 0.01°

- Continuous 360° horizontal rotation, -90° to +90° vertical tilt, completely eliminating blind spots

- Ultra-quiet PTZ drive technology, suitable for noise-sensitive areas

- Surge and lightning protection, effective protection range 2500VA

- Intelligent Temperature Control: Variable power heater, ultra-low temperature startup, automatic cooling fan

- IP67 protection rating

- Small integrated design, total unit weight 8 kg

Product Description

The AK-LF2000 Series Dual-Spectrum Integrated PTZ Camera is a high-performance camera specifically designed and developed for special applications, heat source monitoring, and long-range day/night surveillance.

The product is based on the latest opto-mechatronic integration technologies: 6th generation uncooled infrared imaging technology, high-definition visible light imaging technology, high-speed precision small gimbal technology, edge computing (AI front-end) technology, and high-level field protection technology. This enables the device to "see far," "see clearly," and "understand what it sees" even in harsh outdoor environments.

The thermal imager uses a high-sensitivity, ultra-high resolution uncooled detector, an athermalized compact infrared lens, and advanced image processing algorithms to provide smooth, detailed images. The visible light camera uses an integrated full-HD color/black-and-white dual-mode low-illumination core. The overall unit is small and lightweight, bringing great convenience to user installation and application. It is complemented by a 360° omnidirectional no-blind-spot high-speed PTZ, enabling target search over distances up to 5km. An embedded industrial-grade control system achieves highly stable control of the camera's zoom, focus, mode switching, and PTZ tilt/pan movements.

The AK-LF2000 series housing is made of high-strength aluminum alloy, featuring a robust and fully sealed structure, achieving an IP67 protection rating. The surface is coated for oxidation resistance and anti-salt fog protection, ensuring long-term stable operation in harsh outdoor environments.

Technical Specifications

| Range Indicators |  |
| --- | --- |
| Range against a person | Visible Light: Detection 2000m, Recognition 1000m, Identification 500m Thermal Imaging: Detection 1000m, Recognition 500m, Identification 300m |
| Range against a drone | Visible Light: Detection 1200m, Recognition 600m, Identification 300m Thermal Imaging: Detection 600m, Recognition 300m, Identification 150m |
| Thermal Imaging Specifications |  |
| Detector | 6th Gen Uncooled Detector, Wavelength 7-14μm, Array 640×512 |
| NETD | <35mK (@25°C F1.0) |
| Lens | Fixed focal length, F1.2, 50mm |
| Lens Control | Manual focus / Auto-focus (selectable) |
| Image Processing | 16 pseudo-color images, Black-hot/White-hot polarity switching, Uniformity correction |
| Burn Protection | Supports anti-sun burn protection |
| Temperature Measurement | Supports multi-point temperature display, threshold setting, and alarm function |
| Visible Light Specifications |  |
| Image Sensor | 1/1.8" Low Illumination CMOS |
| Pixels | 4 million |
| Lens | 6.5mm ~ 240mm |
| Minimum Illumination | Color 0.01Lux (F1.5), Black/White 0.001Lux (F1.5) |
| Defog | Electronic + Optical Dual Defog |
| Image Stabilization | Supports gyroscope-based electronic stabilization, heat haze suppression |
| Image Processing | Supports 3D intelligent noise reduction, highlight suppression, wide dynamic range, frame accumulation, backlight compensation |
| Operation Specifications |  |
| Pan/Tilt Speed | Control Speed: 0.01°/sec to 60°/sec, Preset Speed: 100°/sec |
| Speed Levels | 64 levels, supports stepless speed control (10000 levels) |
| Rotation Range | Horizontal 360° continuous rotation, Vertical +90° to -90° |
| Presets | 255 preset positions (expandable to 10000), power-off memory, not lost or deviated after power-off |
| Precision | ±0.1° |
| Positioning Function | Supports absolute positioning, relative positioning, and GPS information positioning |
| 3D Positioning | Supports box selection, zooming, and positioning |
| Data Feedback | Query/Active upload of azimuth angle, field of view, focal length value, focus value |
| Integration Methods | Transparent channel, SDK protocol, private protocol, National Standard GB28181, ONVIF, compatible with mainstream border/coastal defense platform software |
| Interface Specifications |  |
| Network Interface | 1 x 100M Ethernet port |
| Serial Interface | 1 x RS422 interface |
| Power Interface | DC24V, Max power consumption 45W, Power consumption without heater 15W |
| Environmental Specifications |  |
| Manual Defog/Defrost | On/Off |
| Operating Temperature | -40°C ~ 70°C |
| Storage Temperature | -60°C ~ 90°C |
| External Environment | Humidity ≤95% RH (non-condensing), operable at altitudes up to 3500m |
| Temperature Control System | Intelligent temperature control, variable power heater, ultra-low temperature startup, automatic cooling fan |
| Wind Resistance | Normal operation at wind speeds up to 150 km/h, withstands maximum wind speeds up to 210 km/h |
| Lightning & Surge Protection | Power interface 6000V, Signal interface 4500V |
| Protection Rating | IP67 |
| Airtightness | Maintains pressure for extended period after filling with high-pressure gas |
| Dimensions | 426mm × 210mm × 236mm (L × W × H) |
| Weight | Net weight 8KG |

Product Dimension Drawing